2022.04.26

------------------------------------------------------------------------------------------------

Assertion Progress

------------------------------------------------------------------------------------------------

- 這週進度先按照一個流水線標準的 hazard handling 寫 assertion

- 這部分完成後會依照正常時候的功能運作寫 assertion

  - now → if (signal) ? hazard_handling : 1

    - 因為 always block 是 clk-depending 的, 每個 clk 會有不同值 assign 進來, 所以 if (!signal), 如果要寫"reg[address] == some_data", 必須寫在 always block 裡面並用"assert (<expression>)", 而不是寫在 module 裡面, 這部分尚未實作

    - 要寫"reg[address] == some_data"的困難處一是設計時由於 data 操作的方便性, designer 可能會把 data 做一些處理, 而不會單純該 reg 為甚麼值就是甚麼值, 所以可能如下圖, instruction 應該為 instru[31:0], 但因為一些操作, 讓內部的 immediate, rs, rd, func7, func3, opcode 等順序反過來 (e.g. @ stage 4)

  - modify to → if (signal) ? hazard_handling : 一般操作

  - 並加上其他一般功能

  - currently no assertion for cache operation

- 再來就是為避免被 src code 的判斷式影響, 寫更 hierarchy 高的 assertion 去包他

- 依舊用 yosys assertion, 因為方便 parse 完後不用轉格式, 轉變數, 直接寫在想要 assert 的地方即可

- 感覺到時候寫 assertion 不太像是一個太可依賴的辦法, 因為把重要的 handling 或判斷式 assert 下去, 加上變數統一配合 design 使用, 其實跟原本 design 的寫法跟判斷的東西基本上沒差多少, 所以感覺 design 寫好, assert 也寫完了 (?)

------------------------------------------------------------------------------------------------

Assertion Note

------------------------------------------------------------------------------------------------

- 因為粗略的 assertion 是按照正規步驟, 以及按照原始碼自訂的變數去 handle if hazard 等發生如何處理, 這些大致跟 Jeter 的 code 一樣, 所以如果要 insert bug, 可能要考慮 insert 繞多圈一點的 bug, 否則可能要嘛偵測不到 forwarding bug (assertion 除非寫到這麼細), 要嘛直接被抓到 (assertion 包含該判斷式), 或是我 assertion 再想怎麼讓他更 hierarchy 而不是照著正規步驟 handle

- ALU, jump 等細部 ALUop 操作假定都正確的 (?), i.e. 這些較低級的 bug 不會是用我們的方法偵測出來, 因為一般的 tb 就可以抓到了

- 如果 assertion 會用到 internal reg, wire 的話, assert 會寫在 pipeline_stageX.v 的檔案中, 如果只會用到該 module 的 I/O, 則將他寫在 global 的 RISCV_pipeline.v 中

------------------------------------------------------------------------------------------------

Assertion Implementation @ golden RISCV

(Consider CHIP.v > RISCV_pipeline.v, no cache operation)

------------------------------------------------------------------------------------------------

Opcode Spec

Stage 1 (instruction fetch)

[ branch & flush ]

[ memory stall ]

[ load-use hazard ]

- Data Hazard, “Load” 指令需在 stage 4 才能從 memory access

[ if no special case ]

- parameters will directly pass to stage 2, so no need assertion

  - PC_1, instruction_1, prev_taken_1

[Stage 1 Simulation]

→ cd Baseline/src

→ read -sv ALUPipeline/RISCV_pipeline.v

→ hierarchy -top RISCV_Pipeline

→ prep -top RISCV_Pipeline

→ sim -vcd debug_temp/test.fst (no bug is detected)

Stage 2 (instruction decode)

[ flush ]

[ memory stall ]

[ data hazard → load-use hazard → handle by “interlock” ]

[ data hazard → need undefined value → handle by “forwarding” ]

(stage 5: drag wire to write_back to register_bank)

[ control unit ]

[ if no special case ]

- parameter will directly pass to stage 3, so no need assertion

[Stage 2 Simulation]

→ cd Baseline/src

→ read -sv ALUPipeline/RISCV_pipeline.v

→ hierarchy -top RISCV_Pipeline

→ prep -top RISCV_Pipeline

→ sim -vcd debug_temp/test.fst (no bug is detected)

Stage 3 (execute)

[ memory stall ]

[ forwarding unit ]

(consider data/reg/signal in MEM or in WB)

[ if no special case ]

- 根據不同的 decode 指令結果做不同的運算

[Stage 3 Simulation]

→ cd Baseline/src

→ read -sv ALUPipeline/RISCV_pipeline.v

→ hierarchy -top RISCV_Pipeline

→ prep -top RISCV_Pipeline

→ sim -vcd debug_temp/test.fst (no bug is detected)

Stage 4 (memory access)

[memory stall]

[ if nothing special ]

- 按照 signal 指令賦值 (load, store)

[Stage 4 Simulation]

→ cd Baseline/src

→ read -sv ALUPipeline/RISCV_pipeline.v

→ hierarchy -top RISCV_Pipeline

→ prep -top RISCV_Pipeline

→ sim -vcd debug_temp/test.fst (no bug is detected)

Stage 5 (writeback)

[ Mem2Reg signal to control write_back_data ]

(ALU_result 的 data 有被處理過, 應該不是這麼簡單直接讓 assert (writeback_data_5 == ALU_result)) → 因為這會噴 assertion error

[Stage 5 Simulation]

→ cd Baseline/src

→ read -sv ALUPipeline/RISCV_pipeline.v

→ hierarchy -top RISCV_Pipeline

→ prep -top RISCV_Pipeline

→ sim -vcd debug_temp/test.fst (no bug is detected)

------------------------------------------------------------------------------------------------

RISCV Reference

------------------------------------------------------------------------------------------------

- RISC-V formal checking ref:

  - https://github.com/SymbioticEDA/riscv-formal

- ALU pipeline 5 stage to figure out some assertions:

  - https://en.wikipedia.org/wiki/Classic_RISC_pipeline

  - (*) https://blog.csdn.net/whenloce/article/details/87071341

- Stage 3 (Execution) → forwarding unit

  - https://reurl.cc/Er2bl1

2022.04.19

------------------------------------------------------------------------------------------------

Assertion 實驗目的

------------------------------------------------------------------------------------------------

- 熟悉 assertion 語法如何在 yosys 中運作

- 針對我們的 CPU (golden) design, 寫一些必須的 assertion 去確定理論上沒有 bug (雖然跑過 tb 是沒有問題的)

- 針對我們的 CPU (insert bug) design, 看會不會被基本的 assertion 打到, 如果會, 那 bug 安插得太簡單了, 改掉 ; 如果不會, 那可以試著寫更深的 assertion, 看怎麼樣可以打到我們的 bug

  - 當然, 若是有 assertion 打到這個 bug, 除了看合不合理以外, 也要加入到 CPU (golden) design 的 assertion, 如果

------------------------------------------------------------------------------------------------

yoysy 支援的 assertion format

------------------------------------------------------------------------------------------------

- yosys 支援的 SVA (SystemVerilog Assertion) 為"最基本的 format"

  - 在 module block 裡面, 沒有 $display, 單純 “assert property (event);”

  - 在 always block 裡面, 用 “assert (event);”

  - (o) e.g.

- 應該無法用額外新開的檔案 assertion.svh 寫入所有 assertion property, 然後再回到 module block 裡面直接 include 這個檔案 → 有空試試看

  - tutorial: https://www.systemverilog.io/sva-basics

  - (?)

------------------------------------------------------------------------------------------------

Exp1: assert property 可以 work

------------------------------------------------------------------------------------------------

- 在 CHIP.v 的 CHIP module 中, assert 一個恆 false 的 property → (0 == 1), 然後執行下面步驟

- 在 design hierarchy 可以看到 assert 會被 yosys parsing 出來

- 當 sim 完後會發現每個 cycle 都能抓到這個 assertion 的 bug, 所以代表可以 work

------------------------------------------------------------------------------------------------

Exp2: 在必要 module 塞 basic assertion (w/o bug version)

→ 確保在一般情況下不會有問題 (shallow assertion)

------------------------------------------------------------------------------------------------

- Final.v (Final_tb.v 簡化版) → Final

  - CHIP.v → CHIP  (file → module 架構)

    - cache_2way.v → cache (for I$ and D$)

    - RISCV_pipeline.v → RISCV_Pipeline

      - 2way_BTB.v → BTB

      - stage1.v → instruction_fetch

      - stage2.v → instruction_decode

      - stage3.v → Execution

      - stage4.v → Memory

      - stage5.v → Writeback

      - 5 stage detail ref: https://reurl.cc/anlZo3

  - TestBed_hasHazard.v → TestBed

  - slow_memory.v → slow_memory

------------------------------------------------------------------------------------------------

Exp3: 針對 write 寫 assertion (w/o vs. w/ bug version)

------------------------------------------------------------------------------------------------

- ref: https://reurl.cc/VDazYQ

([*2] → 連續兩個 clk 都成立 ? or sequence 內要測兩個 signal)